Measurement of differences in estimation of chewing strength for foods with different hardness

Delwar Hossain 1,  Edward Sazonov 1*

Department of Electrical and Computer Engineering, The University of Alabama

*	Correspondence: esazonov@eng.ua.edu

Abstract:  This paper presents a proof of concept for detecting differences in chewing strength while eating foods with different hardness (carrot as a hard, apple as moderate and banana as soft food) . The goals of this study are to explore the feasibility to estimate chewing strength using non-invasive wearable sensors and to compare sensor modalities which may be used for estimation of chewing strength. Four wearable sensor systems were evaluated. They are: (1) a gas pressure sensor measuring changes in ear pressure proportional to ear canal deformation during chewing; (2) a flexible, curved bend sensor attached to right temple of eyeglass measuring the contraction of the temporalis muscle; (3) a piezoelectric strain sensor placed on the temporalis muscle, and (4) electromyography sensor with electrodes placed on the temporalis muscle. Data from 15 participants wearing all four sensors at once were collected. Each participant took and consumed 10 bites of carrot, apple, and banana. The hardness of foods were measured by a food penetrometer. Single factor ANOVA found a significant effect of food hardness on the standard deviation of signals for all four sensors (P-value < .001). Tukey’s multiple comparison test with 5% significance level, confirmed that the mean of the standard deviations are significantly different for the provided test foods for all four sensors. Results of this study indicate that wearable sensors may potentially be used for measuring chewing strength and assessing the food hardness.

Keywords: mastication; chewing; chewing strength; food hardness; chewing sensor; electromyography; strain sensor; pressure sensor.

1. Introduction:

Mastication is a very complex process which depends upon many factors. Physical properties of food such as hardness, flavor, appearance, size etc. play an important role in the selection of food. The masticatory function is related to sensory-motor activities involving teeth, muscles of jaw, lips, tongue, cheeks, and the salivary secretions production. This is why, the interaction between food perception, oral physiology, and food properties is an intriguing field of research. The perceived food texture or hardness is one of the three main essential factors (appearance, flavor and texture) of food. Previous research brought into attention that food texture influences the chewing process, because changes in food texture or hardness cause masticatory changes (Bourne, 2004; Mioche, 2004). Most of the previous studies which were performed to investigate chewing strength change due to change in food hardness, used artificial and nonfood test materials instead of natural food (Foster et al., 2006; Kohyama et al., 2004; Mioche and Peyron, 1995; Paphangkorakit and Osborn, 1998; Peyron et al., 2002; Tarkowska et al., 2017). In(Kohyama et al., 2004), authors used silicon rubber as model food material to investigate effects of sample hardness on human chewing force. In (Peyron et al., 2002) authors used four separate visco-elastic model foods which were identical in shape and size but different in hardness to examine effects of increased hardness on jaw movement. However, no artificial characterization of food hardness can completely simulate the complex oral process during chewing (Heath and Lucas, 1987). One notable difference is that within the first few chews of real food, mechanical properties change drastically. The time require to change the properties of real food is very short (as little as 0.2s to 0.3 s of first chew)(Kohyama and Nishi, 1997), thus, detecting these changes depends on the sensitivity of the sensors used to measure chewing strength. In this study, we aim to clarify the relationship between estimation of chewing strength measured by a variety of sensors and food hardness by conducting a study among healthy individuals consuming real food.

In order to analyze the relationship of chewing strength estimation and food hardness or texture, several sensor modalities have been used. Researchers were using Gnathodynamometer to measure bite force for a long time (Rane et al., 2017; Rowlett, n.d.). Dynamometer is another mean for estimation of chewing force (Guimaraes et al., 2007; Kogawa et al., 2006; Palinkas et al., 2010). Another way of accurately estimating the chewing strength, is using electrical strain gauges(Anderson, 1953; Fernandes et al., 2003; Howell and Brudevold, 1950; Howell and Manly, 1948; Xu et al., 2008). In order to find the mean of maximum bite force in adults with normal occlusion, a strain gauge force transducer was developed (Bonakdarchian et al., 2009) and positioned between first molars. In(Ferrario et al., 2004), authors used strain-gauge bite transducer for assessment of chewing force on single tooth. More recently researchers used strain-gauge to develop a low cost bite force measurement device (Fastier-Wooller et al., 2016). A multiple point pressure sensor sheet was used to detect the biting pressure for crackers (Kohyama and Nishi, 1997), raw carrots and fish gels (Kohyama et al., 2005), and cucumbers (Kohyama et al., 2013). Force Sensitive Resistor (FSR) is another common sensor, which is becoming popular for measurement of bite force (Bousdras et al., 2006; Diaz Lantada et al., 2012) . The off the shelf availability, low cost and ease of use of FSR are the reasons for their wide application in chewing strength estimation and bite force measurement (Flanagan, 2017; Lin et al., 2009, 2011). A combination of sensors is also used by the researchers to record bite force. Recently both FSR and strain gauge were used to develop a novel human bite force recorder (Khaghaninejad et al., 2017). Surface EMG is another type of sensor which is being used by researchers  for estimation of chewing strength and bite force measurement (Kohyama et al., 2005; Shimada et al., 2012; Steiner et al., 2009)  These devices were positioned inside mouth, placed on tooth and jaws, which might interfere or disturb the smooth movements of jaw during chewing. Over the past 40 years several methods have been proposed to evaluate the chewing strength and bite force in mastication process using different chewing simulators (Kohyama et al., 2008; Korioth and Versluis, 1997; Salles et al., 2007). But the complex phenomena of intra-oral process in chewing cannot be fully orchestrated by any simulator yet. In this study, four wearable sensor systems were used which do not interfere with jaw movement during chewing, making the estimation of chewing strength differences among foods with different hardness more reliable.

In this recent era of wearable devices, several approaches were developed for detection of chewing both in laboratory and free living environment (Dong et al., 2014; Farooq and Sazonov, 2016a; Fontana et al., 2014; Päßler and Fischer, 2014; Rahman et al., 2015). In (Farooq and Sazonov, 2016b) authors developed a wearable device, which can automatically count chews using a piezoelectric film sensor placed below the ear.  The chewing sound captured through an earpiece was used to develop automatic chewing detection in (Päßler and Fischer, 2014). These wearable devices were never explored to detect variation of chewing strength estimation for foods with different hardness. In this study, four wearable sensor modalities were explored for the first time to estimate chewing strength and to explore the relationship between food hardness and estimated chewing strength.

The aim of this study is to explore the possibility of using wearable sensors to estimate strength of chewing real foods. Air pressure sensor, piezoresistive bend sensor, piezoelectric strain sensor and surface EMG sensor are the four sensor modalities used in this study. Using these four wearable sensors, relationship between food hardness and estimated chewing strength was explored.

2.  Materials and Methods

2.1 Test Foods

Three types of natural foods with different textures and properties were used in the study: raw carrots (8-10 inches in length) as hard food, apple (Gala) as food with medium hardness and banana (organic, medium ripe) as soft food.  All these food items were purchased as a single batch from a supermarket (Publix). All foods were preserved in the refrigerator when not in use. The study was performed within two days of the purchase to reduce variation in food properties due to ripening.

2.2 Objective Hardness Evaluation

The hardness of the test foods were measured using a hand-held Magness-Taylor pressure tester known as Fruit Hardness Tester. In this study, GY-3 Fruit Hardness Tester from Vetus Instrument was used. This device has 2 pressure tips (8mm and 11mm diameter) for measuring the hardness of fruit with different texture. The 8mm pressure tip was used to measure the hardness of apple and banana. A custom made 3.5mm diameter pressure tip was used to record the hardness of carrot. Approximately 2mm of skin of apple was peeled. The hardness measurements were made half way between the stem and calyx ends of the fruit. Using the 8mm probe two readings were taken from two sides of the apple. The rest of the fruit was sliced and served to a participant. For carrot, after peeling approximately 2mm of skin in the middle, two reading were taken using the 3.5mm probe. The rest of the carrot was sliced (approximately 0.5 inch) and served to the participant. For banana, two bananas were served to each participant and one reading was taken from each. The banana was cut in half, the skin was peeled and hardness was measured using 8mm probe.  To validate the measurement of custom-made 3.5mm pressure head, a pilot study was performed where 10 samples from 5 apples (2 measurements for each) were taken using both 8mm and 3.5mm pressure tip. A single factor ANOVA (F-value-.03, p-value -0.87) shows that there is no significant difference between 3.5mm and 8mm pressure head readings of hardness of the apples.

2.3 Wearable sensors

In this study, four wearable sensors were used to estimate the chewing strength during consumption of test foods.

2.3.1 Ear Canal Pressure Sensor

This device consists of air pressure sensor (SM9541, from Silicon Microstructures) system and a custom molded earbud (Figure 1a). The sensor measures changes in ear canal pressure during chewing. Commercially available Shirkfin Self-Molding Earbud silicon rubber was used for making of custom-molded earbud of each participant. The data from the sensor was sampled at a rate of 128 Hz by STM32L476 microprocessor and stored in a SD card.

2.3.2 Piezoresistive Bend Sensor

The sensor was implemented as an attachment to the right side of eyeglass frame (Figure 1b). A short piezoresistive sensor sensitive to bending (Spectra Symbol 2.2” sensor) produced a signal proportional to the force applied by the temporalis muscle to the frame of the glasses. The contraction of temporalis muscle during chewing caused the bend sensor to change the resistance. A layer of polyurethane coating was applied to the sensor to protect the sensor from sweat. The device was attached to the temple of the eyeglasses using double sided tape covered with strong adhesive. Data from the sensor was sampled at a rate of 128 Hz by the same microprocessor and stored in a SD card.

2.3.3  Piezoelectric Strain Sensor:

The piezoelectric sensor (LDT0-028K from Measurement Specialties Inc. VA) was attached to the left temporalis muscle by medical tape (Figure 1c). The signal produced by the piezoelectric strain sensor was created by the contraction of the temporalis muscle. The data collected by the sensor was sampled at a rate of 1 Khz using MSP430F2417 processor and sent via Bluetooth to an Android phone for storage. The detailed information about the sensor can be found in [37].

2.3.4 Electromyography Sensor:

The activation of the temporalis muscle can also be recorded using an electromyography (EMG) sensor (Figure 1d). The EMG surface electrodes were placed on right temporalis muscle and the EMG signal was captured by BITalino (r) evaluation board. The data from the sensor were sampled at a rate of 1KHz using ATMega328 microcontroller and sent over Bluetooth to an Android phone for storage.

Figure 1: Four sensor systems: (a) Ear Canal Pressure Sensor (b) Piezoresistive Bend Sensor (c) Piezoelectric Strain Sensor (d) EMG Sensor.

2.4 Participants

A total of 15 participants (8 males, 7 females) aged 24.1±4.7 years, were recruited to participate in this study. The average body mass index was 22.7 ±2.5 kgm2 . Participants did not present any existing tooth and jaw-muscle pain or medical condition that affected chewing. This study was approved by the Institutional Review Board at The University of Alabama. Participants read and signed an informed consent document before participating in the study.

2.5 Data collection

The full experiment consisted of two visits to the laboratory. During the initial visit, participants were briefed about the study and signed informed consent. Ins addition, a custom molded earbud was also made and an eyeglass frame was selected for individual participants. The placement of bend sensor on the frame was also determined in initial visit. In the second visit, upon arrival the wearable sensors were given to the participant to wear, all at the same time. The participants consumed all of the food items while seated. Before serving each food item, a measurement of food hardness was recorded using fruit hardness tester. The participants took at least 10 bites of each food item. The subjects were asked to chew in a natural and self-determined pace on their preferred chewing side. The sequence of test foods was randomly assigned among the subjects; the order of the food items was recorded. Subjects were instructed to provide a pause of approximately 30 seconds in chewing between two consecutive food items.

2.6 Signal processing

The signals recorded by each sensor system were processed offline in MATLAB (Mathworks Inc., Natick, MA, USA). As the characteristic frequencies that define chewing phenomena normally fall within the range of 1.25-2.5 Hz(Sazonov and Fontana, 2012), the recorded pressure sensor, piezoresistive bend sensor and piezoelectric strain sensor signals were filtered using a low pass filter with a cut off frequency of 3 Hz. Raw EMG signal was used for analysis. Each recorded sensor signal was z-normalized using the following equation before processing.

z=x-xS

Where,

x= mean of the sample

S=standard deviation of the sample

After being normalized, each sensor signal was truncated for separate test foods using the pause showed as vertical red line in the Figure 2 and the record of order of served test foods.

Figure 2: Truncation of full signal for separate test food.

Piezoresistive bend sensor, piezoelectric strain sensor and EMG sensors used in this study utilize movement of temporalis muscle while eating to detect chewing. Pressure sensor use the change in air pressure inside ear canal while chewing to detect eating episodes. As the sensors are in contact with the skin or use air canal pressure rather than placed inside the mouth, we can only estimate strength of chewing rather than directly measure the force on the interface between teeth. In(Tortopidis et al., 1998), the authors used standard deviation as an estimator of bite force variability among different position of dental arch. In(Kohyama et al., 2004) authors conducted a study using silicone rubber as model food to find the effects of sample hardness on human chewing force and that standard deviation of chewing cycle is proportional to the Peak force (N) for both incisors and molars. Therefore, the standard deviation of the sensor signal was taken as an estimate of the chewing force in this study.

Only the first ten bites of each test food item for an individual subject were used for analysis. For each bite, the standard deviation of the signal for the first five chews was calculated and used as a metric to estimate chewing strength for foods with different hardness. The chewing strength estimated from the standard deviation of the first five chew of each bite is an observation point. In this study, for an individual subject, 10 observation points for each test food item were used. This study comprises a total of 15 subjects which results 150 observation points for an individual test food item. Thus, a total of 150 data points were collected for each test food item.

2.7 Statistical Analysis

Statistical analysis was performed using MATLAB (ver. 2018b for windows) (MathWorks, Inc.). A single factor analysis of variance was performed to test the effect of sample hardness on the chewing strength. When the hardness effect was significant (P<.001), Tukeys multiple comparison test was applied as post-hoc analysis. The null hypothesis that, there is no significant variation in chewing strength estimation among test foods against the alternative hypothesis, there is significant variation in chewing strength estimation among test foods is tested using a Single-Factor ANOVA. A post-hoc analysis of variation (Tukey’s test) was performed to find out whether the variation is significant or not among the estimator of chewing strength for different test foods after rejecting the null hypothesis.

3. Results

3.1 Objective hardness evaluation

Objective hardness of the test foods was measured using GY-3 fruit penetrometer before serving the foods to individual subject. For each test food item, two readings were taken and an average reading was recorded.

Table I: Objective hardness measurement record of test foods

| Subject | Carrot hardness (kgcm2) | Apple hardness (kgcm2) | Banana hardness (kgcm2) |
| --- | --- | --- | --- |
| 01 | 29.5 | 6.75 | 1.3 |
| 02 | 29.8 | 6.35 | 1.1 |
| 03 | 27.4 | 6.5 | 1.2 |
| 04 | 30.8 | 6.25 | 1.3 |
| 05 | 30 | 7 | 1.35 |
| 06 | 29.5 | 6.2 | 1.2 |
| 07 | 29.25 | 6.6 | 1.35 |
| 08 | 30 | 6.6 | 1.2 |
| 09 | 28 | 6.75 | 1.5 |
| 10 | 30.3 | 6.5 | 1.3 |
| 11 | 31.3 | 7 | 1.2 |
| 12 | 28.75 | 6.1 | 1.1 |
| 13 | 30 | 6.6 | 1.1 |
| 14 | 26.6 | 6.35 | 1.3 |
| 15 | 30 | 6.1 | 1.2 |
|  | Mean = 29.4 | Mean = 6.5 | Mean = 1.3 |
|  | Std = 1.3 | Std =0.3 | Std = 0.1 |

Table I, represents the records of objective hardness measurement of test foods. The average objective hardness measured for carrot is 29.4±1.3 kgcm-2, for apple is 6.5±0.3 kgcm-2 and for banana is 1.3±0.1 kgcm-2. The small standard deviation in readings for each test food objective hardness measurement suggests that there was no significant variation in the hardness of similar food served to participants. Again, the variation of objective hardness among different test foods is significant.

3.2 Pressure Sensor

(a)

(b)

(c)

(d)

Carrot

Apple

Banana

Figure 3: (a) Normalized pressure sensor signal (b) Signals of First five chews for test foods (c)Variation of standard deviation in chewing test foods (d) Box plot of chewing strength estimation for three test foods.

Pressure sensor signal of the full experiment (eating of carrot, apple and banana) is shown in Figure 3(a). The first five chews of a bite of three different test foods are shown in Figure 3(b). The magnitude of the sensor signal of these three test foods is quite separable. Figure 3(c) represents the variation of chewing estimation for three test foods among all participants. The estimated chewing strength is well separable for different foods. The box plot of the standard deviation of chewing signals represented in the Figure 3(d) also indicates enough variation of chewing strength estimation among test foods.

(a)

(b)

(c)

Figure 4:Post-hoc analysis of variation (Tukey’s test)

The single factor ANOVA analysis with the 150 bites of each test food provides a  p-value, 3.9×10-46 and F-value, 133.3 which ) clearly indicates that there is significant variation of chewing strength among different test foods. This leads to reject the null hypothesis. Tukey’s test shown in Figure 4, also confirms that the variation of estimated chewing strength among test foods is significant enough ( with ∝=.05). The three groups (test foods) have means of chewing strength significantly different from each other. These tests also showed the chewing strength estimated by the pressure sensor is highest for carrot and lowest for banana.

3.3 Piezoresistive bend sensor

(a)

(b)

(c)

(d)

Apple

Carrot

Banana

Figure 5: (a) Normalized Piezoresistive bend sensor signal (b) Signals of First five chews for test foods (c)Variation of standard deviation in chewing test foods  (d) Box plot of chewing strength estimation for three test foods.

Figure 5(a) represents piezoresistive bend sensor signal of full experiment. When there is no chewing, the sensor signal has values around zero. While chewing, the sensor signal oscillates which represents the movement of temporalis muscle. Figure 5(b), represents the signals of first five chews of a bite for three different test foods. From the signal, the variation in estimation of chewing strength is quite visible. Carrot has higher peak to peak amplitude, apple has moderate and banana has the lowest amplitude. Figure 5(c) represents the variation of standard deviation of the signals for all 15 participants. Figure 5(d), also represents the box plot of standard deviation readings of 150 data points for each test food item. These figures indicate that chewing strength estimated by the piezoresistive bend sensor for foods with different hardness is well separable.

(a)

(b)

(c)

Figure 6: Variation among groups using Tukey’s test

Single Factor ANOVA analysis with the 150 data points for each test food provides a p-value of 1.8×10-77 and F-value of 269.3, which suggests that there is significant variation of chewing strength estimation  among different test foods. Tukey’s test (Figure-b) also confirms that the variation of estimation of chewing strength among test foods is significant enough ( with ∝=.05). The three groups (test foods) have means of estimated chewing strength significantly different from each other. This test also proved that, the estimated chewing strength by piezoresistive bend sensor is highest for carrot, lowest for banana and moderate for apple. That confirms,  this wearable piezoresistive bend sensor can be used for estimation of chewing strength variation among foods with different hardness.

3.4 Piezoelectric Strain Sensor:

(a)

(b)

(c)

(d)

Apple

Carrot

Banana

Figure 7: (a) Normalized strain sensor signal (b) Signals of First five chews for test foods (c)Variation of standard deviation in chewing test foods (d) Box plot of chewing strength estimation for three test foods.

Strain sensor signal in Figure 7(a) represents that this sensor can effectively detect food intake, separate the bites and also can count the number of chews in an eating episode.   The signal of first five chews of three test food items, Figure 7(b), shows that the variation in chewing strength estimation for three different food items is quite visible. Carrot has higher peak to peak amplitude, apple has moderate and banana has the lowest amplitude. This indicates that standard deviation in the first five chews of a bite can be considered as an estimator of chewing strength. Figure 7(c) represents the variation of standard deviation in chewing signal for different test foods for all 15 participants. The boxplot of the readings in Figure 7(d), shows that the means of the standard deviation of chewing signals are different for the test foods.

(a)

(b)

(c)

Figure 8: Variation among groups using Tukey’s test

Single Factor ANOVA using the 150 data points of estimated chewing strength of each test food item provides a very small p-value  (1.9×10-81) and large F-value  (289.8) which  suggests that the strain sensor can identify the variation of chewing strength estimation  among different test foods. Post-hoc analysis by Tukey’s test, Figure 8, also confirms that the variation of estimated chewing strength among test foods is significant enough ( with ∝=.05). The three groups (test foods) have means of chewing strength estimation significantly different from each other. This test also showed that variation in estimated chewing strength from strain sensor is highest for carrot, lowest for banana and moderate for apple. These tests confirm that this wearable strain sensor can be used for estimation of chewing strength variation among foods with different hardness.

3.5 EMG sensor:

The surface EMG sensor was used as a base for comparison of proposed wearable sensor performance. The signal of EMG sensor, Figure 9(a), represents that the chewing of each test foods can be successfully identified by this sensor. Also from the signals of the EMG, each bite can be separated and number of chews can be counted. The signals of first five chews of three test food items, Figure 9(b), shows that the variation in sensor signal for three different food items is quite visible, which confirms that the standard deviation in the first five chews of a bite can be considered as an estimator of chewing strength for EMG sensor. The variation of standard deviation in chewing different test foods for all 15 participants in Figure 9(c) and the boxplot of the readings in Figure 10, indicates that the surface EMG sensor can effectively record the differences in estimation of chewing strength for the three different test foods.

(a)

(b)

(c)

(d)

Apple

Carrot

Banana

Figure 9: (a) Normalized EMG sensor signal (b) Signals of First five chews for test foods (c)Variation of standard deviation in chewing test foods (d) Box plot of chewing strength estimation for three test foods.

Again the null hypothesis that there is no significant variation in chewing strength estimationamong test foods against the alternative hypothesis, there is significant variation in chewing variable among test foods is tested using a Single-Factor ANOVA for EMG. A post-hoc analysis of variation (Tukey’s test) was performed to find out whether the variation is significant or not among the mean of estimated chewing strength for different test foods after rejecting the null hypothesis.

(a)

(b)

(c)

Figure 10: Variation among groups using Tukey’s test

Single factor ANOVA analysis using the estimated chewing strength of 150 bites of each test food provides a small p-value (2.9×10-91) and large F-value (344.6) which clearly indicates that there is significant variation of chewing strength estimation among different test foods. Tukey’s test (Figure 10) also confirms that the variation of estimation of chewing strength among test foods is significant enough ( with ∝=.05). The three groups (test foods) have means of estimated chewing strength significantly different from each other. This test also showed the estimated chewing strength from EMG sensor is highest for carrot and lowest for banana and moderate for apple.

4. Discussion:

Wearable sensors were never explored to detect and record chewing strength. To the best of our knowledge, in this study 4 different wearable sensor modalities were tested for the first time to estimate chewing strength while chewing three different test foods with hardness significantly different from each other. Four different wearable sensors were developed to detect chewing using temporalis muscle movement (piezoresistive bend sensor, piezoelectric strain sensor and EMG sensor) and change in air pressure in the ear canal (pressure sensor) while eating.

The signals of the sensors clearly indicate that all four sensor modalities were capable of recording chewing phenomena successfully.  As the mechanical properties of food change dramatically within short period of time of first chew, only the first five chews of a bite were used in this study. The signals of first five chews of a bite for all four sensors showed that there is variation of among signals for three different test foods. In this study the standard deviation of first five chews of a bite was considered as an estimator of chewing strength. Analysis of Variation using the estimated chewing strength for different test foods showed that all four sensor system can effectively (p-value<.001) identify the variation in estimated chewing strength because of difference in texture of test foods. The post-hoc Tukey’s test showed that the variation in chewing strength estimation is significant to categorize these test foods into three different categories (soft, medium and hard food). Results showed that the estimated chewing strength is proportional to the food hardness.

Table II: Summarized result of Single Factor ANOVA

| Sensor Type | P-Value | F-Value |
| --- | --- | --- |
| Pressure Sesnor | 3.9×10-46 | 133.3 |
| Piezoresistive bend Sensor | 1.8×10-77 | 269.3 |
| Piezoelectric Strain Sensor | 1.9×10-81 | 289.9 |
| EMG | 2.9×10-91 | 344.5 |

The result of Single Factor ANOVA shows that, EMG performs best in finding variation in estimation of chewing strength for different test foods. But the P-values for other sensors are so small and F-values are quite large which indicates that their performance in finding variation of estimation of chewing strength are  comparable with EMG.

A limitation of this study is that the chewing strength was estimated rather than measured directly. However, use of direct measurement would either make the cost dramatically higher (if instrumenting the teeth) or prevent the use of real foods (use of food substitutes instrumented with sensors). A contribution of the proposed aprroach is that it allows to conduct inexpensive studies with real foods.

In the future, findings of this study will be helpful to develop a predictive model to identify food hardness from estimated chewing strength using wearable sensors. Identifying physical properties of food (hardness) using wearable senosors may provide valuable information (mass, caloric value) to build more robust model of energy estimation in the wild. Relationship between chewing number, chewing duration and chewing time with food hardness can be explored in the future using these wearable sensors.

5. Conclusions:

In order to record bite force or estimate chewing strength while chewing foods with different hardness, wearable sensors were never explored.This paper presented an idea of detection of variation in chewing strength for foods with different hardness using wearable sensor systems. The proposed pressure sensor, piezoresistive bend sensor attached to temporalis muscle, the piezoelectric strain sensor and EMG sensor are feasible to estimate the chewing strength in masticatory process. All four sensor modalities can effectively detect the variation in chewing strength while eating foods with different texture. The estimated strength of chewing is proprtional to the hardness of food. Hard food provides larger value of estimated chewing strength, while soft food results smaller value in chewing strength estimation. The detection of variation in chewing strength estimation for different foods using these low cost, ease of use devices, may be useful for understanding the complex food intakeprocess.

5. Reference:

Anderson, D.J., 1953. A Method of Recording Masticatory Loads. Journal of Dental Research 32, 785–789. https://doi.org/10.1177/00220345530320060601

Bonakdarchian, M., Askari, N., Askari, M., 2009. Effect of face form on maximal molar bite force with natural dentition. Archives of Oral Biology 54, 201–204. https://doi.org/10.1016/j.archoralbio.2008.11.009

Bourne, M., 2004. Relation Between Texture and Mastication. Journal of Texture Studies 35, 125–143. https://doi.org/10.1111/j.1745-4603.2004.tb00829.x

Bousdras, V.A., Cunningham, J.L., Ferguson-Pell, M., Bamber, M.A., Sindet-Pedersen, S., Blunn, G., Goodship, A.E., 2006. A novel approach to bite force measurements in a porcine model in vivo. International Journal of Oral and Maxillofacial Surgery 35, 663–667. https://doi.org/10.1016/j.ijom.2006.01.023

Diaz Lantada, A., González Bris, C., Lafont Morgado, P., Sanz Maudes, J., 2012. Novel System for Bite-Force Sensing and Monitoring Based on Magnetic Near Field Communication. Sensors 12, 11544–11558. https://doi.org/10.3390/s120911544

Dong, Y., Scisco, J., Wilson, M., Muth, E., Hoover, A., 2014. Detecting Periods of Eating During Free-Living by Tracking Wrist Motion. IEEE Journal of Biomedical and Health Informatics 18, 1253–1260. https://doi.org/10.1109/JBHI.2013.2282471

Farooq, M., Sazonov, E., 2016a. A Novel Wearable Device for Food Intake and Physical Activity Recognition. Sensors 16, 1067. https://doi.org/10.3390/s16071067

Farooq, M., Sazonov, E., 2016b. Automatic Measurement of Chew Count and Chewing Rate during Food Intake. Electronics 5, 62. https://doi.org/10.3390/electronics5040062

Fastier-Wooller, J., Phan, H.-P., Dinh, T., Nguyen, T.-K., Cameron, A., Öchsner, A., Dao, D.V., 2016. Novel Low-Cost Sensor for Human Bite Force Measurement. Sensors 16, 1244. https://doi.org/10.3390/s16081244

Fernandes, C.P., Glantz, P.-O.J., Svensson, S.A., Bergmark, A., 2003. A novel sensor for bite force determinations. Dental Materials 19, 118–126. https://doi.org/10.1016/S0109-5641(02)00020-9

Ferrario, V.F., Sforza, C., Serrao, G., Dellavia, C., Tartaglia, G.M., 2004. Single tooth bite forces in healthy young adults. Journal of Oral Rehabilitation 31, 18–22. https://doi.org/10.1046/j.0305-182X.2003.01179.x

Flanagan, D., 2017. Bite force and dental implant treatment: a short review. Med Devices (Auckl) 10, 141–148. https://doi.org/10.2147/MDER.S130314

Fontana, J.M., Farooq, M., Sazonov, E., 2014. Automatic Ingestion Monitor: A Novel Wearable Device for Monitoring of Ingestive Behavior. IEEE Transactions on Biomedical Engineering 61, 1772–1779. https://doi.org/10.1109/TBME.2014.2306773

Foster, K.D., Woda, A., Peyron, M.A., 2006. Effect of Texture of Plastic and Elastic Model Foods on the Parameters of Mastication. Journal of Neurophysiology 95, 3469–3479. https://doi.org/10.1152/jn.01003.2005

Guimaraes, A.S., Carlsson, G.E., Marie, S.K.N., 2007. Bite force and handgrip force in patients with molecular diagnosis of myotonic dystrophy. Journal of Oral Rehabilitation 34, 195–200. https://doi.org/10.1111/j.1365-2842.2006.01665.x

Heath, M.R., Lucas, P.W., 1987. MASTICATION: THE NEED FOR COLLABORATIVE RESEARCH. Journal of Texture Studies 18, 111–123. https://doi.org/10.1111/j.1745-4603.1987.tb00573.x

Howell, A.H., Brudevold, F., 1950. Vertical Forces Used During Chewing of Food. Journal of Dental Research 29, 133–136. https://doi.org/10.1177/00220345500290020401

Howell, A.H., Manly, R.S., 1948. An Electronic Strain Gauge for Measuring Oral Forces. Journal of Dental Research 27, 705–712. https://doi.org/10.1177/00220345480270060701

Khaghaninejad, M.S., Peyravi, A., Khosravifard, A., Peyravi, E., Eftekharian, H.R., Peyravi, M.R., 2017. Novel User-friendly Device for Human Bite Force Measurement. Journal of Dental Biomaterials 4, 475-483–483.

Kogawa, E.M., Calderon, P.S., Lauris, J.R.P., Araujo, C.R.P., Conti, P.C.R., 2006. Evaluation of maximal bite force in temporomandibular disorders patients. Journal of Oral Rehabilitation 33, 559–565. https://doi.org/10.1111/j.1365-2842.2006.01619.x

Kohyama, K., Hatakeyama, E., Dan, H., Sasaki, T., 2005. Effects of Sample Thickness on Bite Force for Raw Carrots and Fish Gels. Journal of Texture Studies 36, 157–173. https://doi.org/10.1111/j.1745-4603.2005.00009.x

Kohyama, K., Hatakeyama, E., Sasaki, T., Dan, H., Azuma, T., Karita, K., 2004. Effects of sample hardness on human chewing force: a model study using silicone rubber. Archives of Oral Biology 49, 805–816. https://doi.org/10.1016/j.archoralbio.2004.04.006

Kohyama, K., Kato‐Nagata, A., Shimada, H., Kazami, Y., Hayakawa, F., 2013. Texture of Sliced Cucumbers Measured by Subjective Human-Bite and Objective Instrumental Tests. Journal of Texture Studies 44, 1–11. https://doi.org/10.1111/j.1745-4603.2012.00360.x

Kohyama, K., Nishi, M., 1997. Direct Measurement of Biting Pressures for Crackers Using a Multiple-Point Sheet Sensor. Journal of Texture Studies 28, 605–617. https://doi.org/10.1111/j.1745-4603.1997.tb00141.x

Kohyama, K., Sasaki, T., Hayakawa, F., 2008. Characterization of Food Physical Properties by the Mastication Parameters Measured by Electromyography of the Jaw-Closing Muscles and Mandibular Kinematics in Young Adults. Bioscience, Biotechnology, and Biochemistry 72, 1690–1695. https://doi.org/10.1271/bbb.70769

Korioth, T.W.P., Versluis, A., 1997. Modeling the Mechanical Behavior of the Jaws and Their Related Structures By Finite Element (Fe) Analysis. Critical Reviews in Oral Biology & Medicine 8, 90–104. https://doi.org/10.1177/10454411970080010501

Lin, K., Liu, T., Lin, S., Chang, C., Lin, C., 2009. Occlusive bite force measurement utilizing flexible force sensor array fabricated with low-cost multilayer ceramic capacitors (MLCC), in: TRANSDUCERS 2009 - 2009 International Solid-State Sensors, Actuators and Microsystems Conference. Presented at the TRANSDUCERS 2009 - 2009 International Solid-State Sensors, Actuators and Microsystems Conference, pp. 2238–2241. https://doi.org/10.1109/SENSOR.2009.5285593

Lin, K.-R., Chang, C.-H., Liu, T.-H., Lin, S.-W., Lin, C.-H., 2011. Experimental and numerical estimations into the force distribution on an occlusal surface utilizing a flexible force sensor array. Journal of Biomechanics 44, 1879–1884. https://doi.org/10.1016/j.jbiomech.2011.04.032

Mioche, L., 2004. Mastication and Food Texture Perception: Variation with Age. Journal of Texture Studies 35, 145–158. https://doi.org/10.1111/j.1745-4603.2004.tb00830.x

Mioche, L., Peyron, M.A., 1995. Bite force displayed during assessment of hardness in various texture contexts. Archives of Oral Biology 40, 415–423. https://doi.org/10.1016/0003-9969(94)00190-M

Palinkas, M., Nassar, M.S.P., Cecílio, F.A., Siéssere, S., Semprini, M., Machado-de-Sousa, J.P., Hallak, J.E.C., Regalo, S.C.H., 2010. Age and gender influence on maximal bite force and masticatory muscles thickness. Archives of Oral Biology 55, 797–802. https://doi.org/10.1016/j.archoralbio.2010.06.016

Paphangkorakit, J., Osborn, J.W., 1998. Effects on human maximum bite force of biting on a softer or harder object. Archives of Oral Biology 43, 833–839. https://doi.org/10.1016/S0003-9969(98)00073-9

Päßler, S., Fischer, W., 2014. Food Intake Monitoring: Automated Chew Event Detection in Chewing Sounds. IEEE Journal of Biomedical and Health Informatics 18, 278–289. https://doi.org/10.1109/JBHI.2013.2268663

Peyron, M., Lassauzay, C., Woda, A., 2002. Effects of increased hardness on jaw movement and muscle activity during chewing of visco-elastic model foods. Exp Brain Res 142, 41–51. https://doi.org/10.1007/s00221-001-0916-5

Rahman, S.A., Merck, C., Huang, Y., Kleinberg, S., 2015. Unintrusive Eating Recognition Using Google Glass, in: Proceedings of the 9th International Conference on Pervasive Computing Technologies for Healthcare, PervasiveHealth ’15. ICST (Institute for Computer Sciences, Social-Informatics and Telecommunications Engineering), ICST, Brussels, Belgium, Belgium, pp. 108–111.

Rane, V., Hamde, S., Agrawal, A., 2017. Development of computerized masticatory force measurement system. Journal of Medical Engineering & Technology 41, 65–71. https://doi.org/10.1080/03091902.2016.1218559

Rowlett, A.E., n.d. The Gnathodynamometer and Its Use in Dentistry. Proceedings of the Royal Society of Medicine 9.

Salles, C., Tarrega, A., Mielle, P., Maratray, J., Gorria, P., Liaboeuf, J., Liodenot, J.-J., 2007. Development of a chewing simulator for food breakdown and the analysis of in vitro flavor compound release in a mouth environment. Journal of Food Engineering 82, 189–198. https://doi.org/10.1016/j.jfoodeng.2007.02.008

Sazonov, E.S., Fontana, J.M., 2012. A Sensor System for Automatic Detection of Food Intake Through Non-Invasive Monitoring of Chewing. IEEE Sensors Journal 12, 1340–1348. https://doi.org/10.1109/JSEN.2011.2172411

Shimada, A., Yamabe, Y., Torisu, T., Baad‐Hansen, L., Murata, H., Svensson, P., 2012. Measurement of dynamic bite force during mastication. Journal of Oral Rehabilitation 39, 349–356. https://doi.org/10.1111/j.1365-2842.2011.02278.x

Steiner, M., Mitsias, M.E., Ludwig, K., Kern, M., 2009. In vitro evaluation of a mechanical testing chewing simulator. Dental Materials 25, 494–499. https://doi.org/10.1016/j.dental.2008.09.010

Tarkowska, A., Katzer, L., Ahlers, M.O., 2017. Assessment of masticatory performance by means of a color-changeable chewing gum. Journal of Prosthodontic Research 61, 9–19. https://doi.org/10.1016/j.jpor.2016.04.004

Tortopidis, D., Lyons, M.F., Baxendale, R.H., Gilmour, W.H., 1998. The variability of bite force measurement between sessions, in different positions within the dental arch. Journal of Oral Rehabilitation 25, 681–686. https://doi.org/10.1046/j.1365-2842.1998.00293.x

Xu, X., Wright, P.S., Hector, M.P., Heath, M.R., Ferman, A.M., 2008. Force, Rate and Work Used During Incisor Penetration on Different Textural Foods. Journal of Texture Studies 39, 115–128. https://doi.org/10.1111/j.1745-4603.2008.00133.x